CAN Protocol Session 1

Tổng điểm3/10

Email của người trả lời (vantuand1710@gmail.com) đã được ghi lại khi họ gửi biểu mẫu này.

Name*

Nguyễn Văn Tuân

1/ What does CAN stand for?*

1/1

Custom Automotive Network

Corporate Action Network

Car Access Network

Controller Area Network

2/ Why CAN? (Choose the correct answers)*

0/1

Automotive Domain needed a way for all ECUs to communicate the data among themselves.

Slow speed, so that data can be carefully taken care of.

Synchronous

Highly immune to electrical noise

Point To Point communication, so that data can be transmited directly.

Câu trả lời đúng

Automotive Domain needed a way for all ECUs to communicate the data among themselves.

Highly immune to electrical noise

3/ Which of the below are NOT CAN's characteristics?*

0/1

Parallel interface protocol

Wired Protocol

Need Acknowledgement

Duplex Communication

Fault Confinement

Câu trả lời đúng

Parallel interface protocol

Duplex Communication

4/ Within a single CLASSIC CAN data frame, which data length can be transmitted?*

0/1

10 bytes

5 bits

8 bits

8 bytes

Câu trả lời đúng

8 bits

8 bytes

5/ In industry, Which speed is usually configured for CAN bus?*

1/1

400kbps

500kbps

600kbps

700kbps

6/ In OSI model, which layer does CAN protocol belong too?*

1/1

Application Layer

Presentation Layer

Session Layer

Transport Layer

Network Layer

Data link layer

Physical Layer

7/ Choose the CORRECT statement regarding CAN bus*

0/1

CAN bus uses Dual-Twisted pair wires called Can High and Can Low

Dual-Twisted pair was used because CAN bus is full duplex communication.

in CAN bus, Logic 0 will overwrite Logic 1

No need to have any resistor in CAN bus because the ECUs have already absorbed any the bounce back effect.

Câu trả lời đúng

CAN bus uses Dual-Twisted pair wires called Can High and Can Low

in CAN bus, Logic 0 will overwrite Logic 1

8/ In order to transmit the data from one node to other nodes, which of these are needed to facilitate the transmission*

0/1

CAN Transceiver

CAN Receiver

CAN Controller

CAN Transmitter

CAN Dongle

Câu trả lời đúng

CAN Transceiver

CAN Controller

9/ Choose the FALSE statement*

0/1

When transmitting data, CAN Controller will form the CAN frame received from Microcontroller and the complete CAN frame is given to CAN Transceiver.

After receiving the CAN frame from CAN Receiver, CAN Controller will remove all the controller specific protocol/fields and transfer the data part to Microcontroller

CAN Transmitter and CAN Receiver does not have any SW, all implementations are done by HW

The microcontroller of the ECU does not connect directly with CAN bus

Câu trả lời đúng

After receiving the CAN frame from CAN Receiver, CAN Controller will remove all the controller specific protocol/fields and transfer the data part to Microcontroller

CAN Transmitter and CAN Receiver does not have any SW, all implementations are done by HW

10/ Which of the bellow Message ID can be filter by Filter Value = 0x678 and Mask = 0x345*

0/1

0x250

0x251

0x252

0x253

Câu trả lời đúng

0x250

0x252